ALTENER – VEHICLE SPECIFICATIONS

BUZZ

Motor: 9 kW PMSM, IP67 Sealed (all-weather)

Battery: 11.8 kWh LiFePO4 (LFP)

System Voltage: 72V

Range: 120–140 km (single charge)

Payload: 550 kg

Gross Weight: 1,100 kg

Top Speed (Eco): 30 km/h

Top Speed (Boost): 50 km/h

Drive Modes: Eco / Boost / Auto / Rev

Auto Mode: Smart switching between Eco and Boost

Transmission: 2-Speed Gearbox (Gear Ratios: 14.96 / 8.06)

Charging: Onboard 25A, 5–6 hrs (0–80%)

Battery Cycles: 2,000+

Display: 7.0" Colour Touch LCD

Cluster Features: Live speed, SoC (battery %), range, trip data, fault codes, maintenance reminders

Reverse Camera: Dedicated 4.3" screen (Container variant)

Start System: Keyless Entry + Encrypted Immobilizer

Connectivity: 4G LTE + GNSS (GPS)

OTA Updates: Yes (remote software)

Fleet Management: Live GPS, Geofencing, Diagnostics

Mobile App: Real-time battery %, range estimate, remote immobilizer, trip logs, SOS, push alerts

Web Portal: Live fleet map, driver scores, idle time monitoring, maintenance tracking, CSV/PDF export

Front Brake: Disc Brake

Rear Brake: Drum Brake

Regenerative Braking: Yes

Tyre Size: 10–12 inch heavy-duty

Front Suspension: Twin Shock Absorbers + Springs

Rear Suspension: Rubber Damping + Shock Absorbers

Chassis: High-strength steel

Gradeability: 26.5%

Hill Hold Assist: Yes

Cooling: Active Controller Cooling

LITE

Motor: 6 kW PMSM, IP67 Sealed

Battery: 6.5 kWh LiFePO4 (LFP)

System Voltage: 72V

Range: 100–120 km (single charge)

Payload: 350 kg

Gross Weight: 1,100 kg

Top Speed: 45 km/h

Transmission: Single-Speed Automatic (no gear changing)

Charging: Onboard 25A, 4–5 hrs (0–80%)

Battery Cycles: 2,000+

Display: 5.0" Digital Instrument Cluster

Cluster Features: Live speed, SoC (battery %), range, trip data, fault codes, maintenance reminders

Reverse Camera: Dedicated 4.3" screen (Container variant)

Start System: Physical Key

Connectivity: 4G LTE + GNSS (GPS)

OTA Updates: Yes (remote software)

Fleet Management: Live GPS, Geofencing, Diagnostics

Mobile App: Real-time battery %, range estimate, remote immobilizer, trip logs, SOS, push alerts

Web Portal: Live fleet map, driver scores, idle time monitoring, maintenance tracking, CSV/PDF export

Front Brake: Drum Brake

Rear Brake: Drum Brake

Regenerative Braking: Yes

Tyre Size: 8 inch city-optimized

Front Suspension: Twin Shock Absorbers + Springs

Rear Suspension: Rubber Damping + Shock Absorbers

Chassis: High-strength steel

Cooling: Active Controller Cooling

* Specifications are subject to change without notice.
* Performance values are under standard test conditions.